# VTrans Static Terrestrial LiDAR Scanning

# Operations Manual

Updated 9/9/2021

# Overview

Light Detection and Ranging (LiDAR) is a remote sensing technology that uses different wavelengths of light to measure distance for 3D modeling. The instrument fires and records these light pulses 100,000 to 400,000 times a second. When the light pulse hits an object, the instrument assigns an XYZ coordinate to each point based on the time it takes for the light to hit the target, be reflected, and return to the scanning instrument. The term LiDAR is commonly used interchangeably with laser scanning.

There are two major types of LiDAR systems – airborne laser scanning (ALS) and terrestrial laser scanning (TLS). TLS is performed with either mobile, mounted to moving vehicles, or static, fixed on a stationary tripod, scanning instruments. This manual pertains to the operations with static terrestrial laser scanning (STLS).

Survey specifications describe the methods and procedures needed to attain desired survey accuracy standards. For complete accuracy standards, refer to Chapters 4-6 in the Vermont Agency of Transportation (VTrans) Route Survey Manual. VTrans survey specifications shall be used for all VTrans STLS projects.

# STLS Applications

STLS equipment is available for VTrans project work. Typical scan requests include but are not limited to:

- Roadside Ledge Monitoring Scans

- Earthwork Analysis Scans

- As-Built Structure Scans

- Pavement Surface Scans

- Architectural and Historical Preservation Scans

- Preliminary Data Collection Scans

# Project Selection

The following are factors to consider when planning the use of STLS for each scan request:

- Safety

- Length/size of project

- Detail required

- Forecasted weather and atmospheric condition

- Desired deliverables

- Project time constraints

- Site/structure complexity

- Traffic volumes

# Equipment and Safety

All field equipment used to perform STLS data collection should be appropriately maintained to ensure the accuracy standards required for the project. This determination will be from the stated specifications for the equipment by the manufacturer. Equipment and accessories must be regularly checked for accuracy and proper function as well as for field personnel safety.

#### Scanner Specifications

| TX8 Scanner Specifications |  |  |  |
| --- | --- | --- | --- |
| Standard Range | Field of View | Operating Temp Range | Scan Time per Battery |
| 2 - 360 ft | 360° X 317° | 32°F - 104°F | >2 hours |

See TX8 Detailed Specifications document in the Help Documents Folder on the L:Drive

- Scanner Range

Care should be taken during the project to ensure that point cloud data accuracy is not compromised by measurements outside the useful range of the scanner. Consideration of scanner range should be made during initial survey activities establishing survey control as well as instrument placement between station set ups and in relation to the feature being scanned. Methods for accomplishing this might include the implementation of range and/or intensity filtering during data collection or cutting out any out-of-useful range (noise) data during post-processing.

##### Operation Constraints

The scanner cannot be use in weather where significant precipitation (rain, snow, sleet, etc.) has recently occurred or is forecasted due to surface properties including color, surface reflectivity, surface texture, and angle of incidence can limit scanner useful range. The ideal operating field season should be planned between the April and November calendar months.

##### Scanner Maintenance

The scanner should be sent for servicing through an authorized Trimble distributing partner (ex. Waypoint Technologies) at least once per year, if not more frequently due to reoccurring instrument issues/errors. Scanner diagnostics and firmware will be checked every 6 months to ensure instrument is up to date and correctly operating – see Maintenance section in the TX8 Operator Manual for instruction.

##### Target Requirements

Targets are used as reference points in each station scan to register the project into a cohesive point cloud. Both spherical and flat targets are used in STLS, but spherical targets are preferred for outdoor and larger scan projects because of the greater viewable surface area. The spheres used are made to be exactly 0.75 ft in diameter. Spheres are mounted onto adjustable-height prism pole bipods and set at a fixed height. The size of the target, distance from the scanner, and target scan resolution determine how precisely the target reference locations can be determined. If the distance from the scanner to the target exceeds the manufacturer’s recommended distance, the error may increase dramatically.

#### Field Safety

All field personnel conducting STLS are required to adhere to the VTrans Policy Manual and any PPE guidelines set forth by the Agency to ensure employee and crew safety on VT roadways.

#### Laser Safety

The Trimble TX8 3D scanner is labeled a Class 1 laser device that operates in a laser wavelength of 1.5 μm (invisible light), with a beam divergence of 0.177 mrad Gaussian and an output laser power of 6 kW peak. Never stare into the laser beam or laser beams through magnifying optics. STLS equipment operators should never direct the laser toward personnel operating instrument with magnifying optics such as total stations or levels. Scan crew personnel should follow OSHA Regulation 1926.54 or the manufacturers’ recommendations when using any laser equipment.

# Standards and Procedures

It is each scan crew member’s responsibility to ensure the following procedures are understood and followed to obtain satisfactory results and adhere to VTrans standards.

#### Pre-Scan

##### Project Preparations

Unless a project requires onsite Survey personnel or covers dangerous terrain, only two scan personnel are needed to conduct the scan. The scan date should be planned during fair weather with a rain date in case of inclement weather. If cell service will be limited at the scan site, temperature and atmospheric pressure information should be checked beforehand to have for setting scan parameters.

If traffic control is required to perform the scan, the appropriate district should be contacted at least one week prior to the scan date to schedule – see Maintenance District Map for contact information. It is recommended to avoid scheduling scans on Monday and Fridays if the Districts are needed due to seasonal schedules.

If survey control has been established prior to the scan, obtain and upload the coordinates to the ROWFieldWork web map. This map should be used by scan personnel to locate control in the field and can be accessible through the ESRI Explorer mobile app.

USB storage should be checked prior to use on a scan to make sure old project data has been removed.

##### Equipment

A

BA

E

C

D

A

BA

E

C

D

All equipment information and location should be updated in the Survey Section Asset Inventory spreadsheet.

Scanner Box

- TX8 Scanner & Tribrach (A)

- 4 Charged Batteries (B)

- Project USB (C)

- Power Cords (D&E)

- Measuring Tape

Field Truck

- Dual Clamp Instrument Tripod

- 6 Adjustable height prism pole bipods

- 6 Target Spheres

- Magnetic Locator

- 2 “Survey Field Crew Ahead” Sign Packages

Additional equipment:

- Magnetic Target Mounts

- Survey iPad/Phone

- Adjustable Height Instrument Tripod

- Mag Nails

- Flagging

##### Survey Control

All surveys for control points that will be occupied by a STLS control target shall meet or exceed horizontal and vertical accuracy standards and follow approved methods as described in the VTrans Route Survey Manual. All deviations from these standards are required to be documented as Processing Notes at the time of submission of the scan data.

A minimum of three control points will be set and witnessed throughout the project extents. Best results are typically seen when the control points are evenly spaced horizontally throughout the scan area. Variation in control elevations is also desirable. Points should be accessible enough that an occupied station scan can be performed on each control point - see Occupied Station Scan Set Up section below.

Pavement Surface Analysis or As-Built Structural Scans may require additional or closer spacing of control points as well as shorter maximum scanning distances than other scanning applications.

Asset monitoring scans, where surface movement is being monitored, will require 3 or more permanent control monuments. These monuments should be fixed in ledge and are expected to remain stable for the life of a project, with a minimum service life of 5 years.

All STLS control points shall be provided in Vermont State Plane coordinates and be on the project datum and epoch. RAW files from setting control should either be linked by a Shortcut to their location on the M:Drive or uploaded directly to the Survey folder for the project.

#### Field Operations

All scan personnel are required to read the TX8 Operator Manual prior to operating the laser scanner.

##### Scanner Set Up

If the temperature is below 40°F, the scanner needs to acclimate for a minimum of 15 minutes prior to being powered on. The scanning box should be propped open while equipment is being set up to allow for acclimation. The scanner should always be moved in the scanner box.

USB

Battery

USB

Battery

When installing the battery and USB, the battery should always be put in first, followed by the USB to avoid damaging the USB. When changing the battery, the scanner should be completely turned off and the battery and USB removed in reverse order. If the battery dies mid scan, you will have to re-run the station scan in the same position as the previous station – no targets should be moved.

The scan operator will need to note the current temperature and atmospheric pressure to set the scan parameters for the project. All scan project names should be set to include the programmed project identification number (PIN) if available or town and project name/number along with the scan date in the MMDDYY format (ex. 19V439_092420 or Coventry_147_092420).

Scan mode should be set based off scan density required for the project. Typically, the scan mode is defaulted to Level 2.

The scanner operator should also note the USB’s storage capacity as it appears on the scanner screen. Lengthier scan modes (Level 3 and Extended) or performing numerous area scans can take up more storage on the project USB.

Level

Lock

Level

Lock

At each scan station, the scanner tripod must be set up as high as practical to reduce the angle of incidence and improve the scanner’s effective range and accuracy during the scan. For each station scan  the tripod should be stabilized and leveled prior to mounting and locking the scanner. The scanner should not be moved more than the maximum range identified in the Scanner Specifications section.

##### Target Set Up

All target bipods must be measured at the beginning of the scan. Height should be set to 4.56’ and recorded as part of the scan notes so it can be used for georeferencing.

The height should be measured from the tip of the pole to the top screw piece where the sphere will rest (see circled area in above photo). This height should measure 4.56ft.

Height of these bipod should be checked prior to the start of each scan project.

A minimum of three control target bipods should be set plumb on the verified survey control and must be visible between adjacent scans. These control targets can be used to georeference the point cloud into the Vermont State Plane Coordinate System in the event the scanner cannot be set over the control point. It is advisable to check the plummet position for targets on control at the start of each station scan. All other targets not set on control points should be set at an angle, so they are easily identified during post-processing.

The scan should be conducted in such a manner as to ensure redundancy of the data through overlapping station scans. All six targets must be strategically placed such that targets do not form a linear pattern. A minimum of three coincident targets must be captured in each station scan, so only 1-3 targets should be repositioned between station scans.

Magnetic target mounts can be used in place of bipods on metal structures where 3 or more ground targets exist.

##### Occupied Station (OS) Scan Set Up

To improve georeferencing error in post processing, there should be at least one occupied station (OS) scan performed per verified control being used for the project. If there are 4 verified control points for the project, that means 4 OS scans should be performed.

To set up the scanner for an OS scan, the operator should use the optical plummet on the tribrach to fix the tripod over the control point. Once leveled, the scanner should be mounted and locked into place. Before starting the station scan, the height of the scanner must be measured and inputted into the Scan Parameters.

When the operator is measuring the height of the scanner, they must select the Measurement Method on the Instrument Height screen. Typically, measuring from the Bottom Mark is easiest. Once the height is keyed in, the scanner will make an adjustment for the calculated height of the instrument. The operator can go back to the main screen and perform the station scan.

If the height was not keyed in prior to the scan, it can be keyed in after during the station scan review or by going to the specific station scan of the project and selecting the Setting Gear icon.

It is recommended that the project scan start with an OS scan and end with and OS scan. After the first OS scan, the scanner should be moved off the point and a control target bipod should be reset plumb over the point and vice-versa as the scanner moves closer to the next visible control point.

##### Scan Field Notes

It is recommended to record notes throughout the scan. The ESRI Explorer app allows for documentation based on station scan location and can serve this purpose if the operator so chooses to use it. Survey control points will be uploaded to the app via the ROWFieldWork web map. Notes may include:

- Station Scan #

- Visible control points

- OS Scan Measured Height

- Area scan and Level

- Missed targets

- Error Messages

Scan field notes are required to be submitted for each project.

##### Area Scans

Area scans are useful when the project requires greater detail or on smaller sized projects where moving the scanner between stations is not necessary. Area scans can also increase the point density on targets if the operator is concerned a target is too far away for acceptable capture or is partially obstructed by vegetation. Ledge Asset Monitoring Scans should include at least one area scan, to capture ledge surface feature in the greatest amount of detail. Area scans should be performed when the feature is highly vegetated or very tall.

See TX8 Operator Manual for further instruction on setting up an Area Scan.

##### Scanner Warning Errors

If the scanner returns a warning message after a station scan, the operator should note the error in the station notes.

Level Warning

This warning appears when the scanner position has altered from the initial leveled position prior to running the scan. This is likely a result of vibrations from passing vehicles, moving structural elements that the scanner is set up on, or even strong wind gusts.

To mitigate this warning, the operator should rerun the station scan in the same position as the previous station without moving the tripod or any targets.

Operating Temperature Warning

This warning appears when the scanner is run outside the specified operating temperatures. It is crucial that the scan is planned during fair weather days where temperatures are not below or do not exceed operational temperatures. If temperatures changes are unexpected, the operator should turn the scanner off, return it to the scanning box and either place the box in the truck to warm up or open the battery/USB compartment to release excess heat.

Unformatted USB Error

This warning will appear when the USB is first attached to the scanner. This warning can be dealt with in the field – there is an option to have the scanner reformat the USB in the Settings section on the scanner.

Other Error or Warning Messages

If the scanner experiences any other type of operational error not listed above, the operator should stop the current scan, turn the scanner off, and restart the scanner. If this does not fix the issue, please reach out to the Scan Team Lead to discuss options in the field.

See TX8 Operator Manual for further instruction on pausing or canceling a scan as well as common scanner errors.

#### Post-Scan

All current project data is stored on the aotrowlidar$ instance of the aotrmtsns server (referred to as the L:Drive). Archived project data is located on a backed-up drive on the Survey Processing Workstation. The folder structure standard is documented in the Documents folder of the L:Drive and can be directly copied, pasted, and renamed to create folder for the uploaded project.

##### Project Upload Requirements

Project data will be uploaded to the RAW folder location immediately after the scan. Scanning notes will be entered in the Survey folder along with any site photos, including the required target height check. Notes should be manually entered into the appropriate project folder with the MMDDYY_ScanNotes template. Survey control coordinate are to be uploaded as a .TXT file in the Survey folder.

Scan data files should not be deleted from the Project USB until the project TZF scans have been created, sampled, and the RAW .RWP is saved as a processing file in the Processing folder.

The “Desktop.ini” in the project folder should be opened and only the TOWN – PROJECT NAME text updated to reflect the appropriate project title attributes.

Site photos can be uploaded to the MMDDYY_SitePhotos folder in the Survey folder for the project but must be attached with the online STLS Processing Form.

# Quality Control

Data collected using STLS data should be verified by various means including a statistical comparison of the point cloud to survey validation points, reviewing digital terrain models, checking point cloud redundancy, and comparing elevations of two or more point clouds from overlapping scan areas. It is highly recommended that the previously mentioned validation techniques and data requirements discussed below, are utilized to ensure data quality and accuracy.

### Data Requirements

##### Cloud Sampling

Multiple clouds may need to be produced and sampled for each project. However, as a standard deliverable, the original working or “RAW” Segmented and Classified Clouds (SCCs) must be included – see the Deliverables Section below for details. This raw point cloud is not sampled past initial sampling density (0.01 ft) when the processing file was created. If the only deliverable is a POD Point Cloud, along with the RAW SCCs , a sampled POD for each SCC should also be created and exported. Sampled PODs scan density spacing should not exceed 0.1 ft for structural and ledge features and 0.3 ft for road surfaces. Exceptions can be made to reduce file size or based off specific project request and should be noted in the STLS Processing Form.

If a surface is requested as a deliverable, two PODs should be created of the feature and sampled to 0.5ft and 1.0ft resolution. These two clouds will be used to create the two surfaces, unless a higher resolution is requested. The 0.5ft and 1.0ft surface resolutions have been determined as acceptable for VTrans design standards working in Microstation.

##### Point Cloud Redundancy

Data should be collected so that there is a minimum 5% to 20% overlap (percentage of scanner’s useful range) from one scan to the next adjacent scan. When using cloud to cloud registration, overlap can be as much as 75%.

Redundancy can be controlled by scanner placement throughout a scan. Care should be taken as to no exceed the maximum range of the instrument – see Standards and Procedures section for further details on scanner set up and placement.

##### Registration Methods

Target-based registration should be the primary method used to register project scans. RMS Error cannot exceed 0.02 ft for each individual target, station, and the overall Scan Registration Error. The final generated RTF Registration Report must be saved into the Processing folder of the project.

Cloud to cloud registration is used to register two selected scanning station that share common points in the cloud. This method is typically used when merging an ungeoreferenced cloud with one or more georeferenced clouds or if the cloud does not have to be fixed into Vermont State Plane – see Point Cloud Redundancy section for accepted coincident point overlap.

##### Georeferencing

All projects must be referenced into the NAD83 (2011) Vermont State Plane projected Coordinate System at a minimum of 3 fixed control points. Additional points may be used but not fixed to produce the lowest residual error. Georeferencing Residual Error cannot exceed 0.1ft and error must be recorded in STLS Processing Form. Any exceptions must be documented and reviewed to determine cause of unacceptable error before georeferencing can be applied to the project.

# Deliverables and Documentation

Documentation of scan field work and surveys is an essential part of surveying work. Collecting and processing data not properly documented is susceptible to imbedded mistakes and is difficult to adjust or modify.

### Deliverables

Different projects and requests may require different types of deliverables, which can range from a standard CADD product to various point cloud file types. The typical exports we work with can include:

- LAS/LAZ

- E57

- ASCII

- CSV

- POD

- DXF

- DGN

The standard deliverable packages we produce are as follows:

Point Cloud Only Request

Unless a surface or digitized features are requested, only PODs will be delivered

- RAW SCC PODs

  - Classification groups are:

    - Ground (Project Area)

    - Roadway

    - Utilities (Signs, Utility Poles, Utility Lines)

    - Feature of Interest (Bridge, Culvert, Ledge, etc).

- Sampled SCC PODs

Surface/Digitized Features Requested

- RAW SCC PODs

  - Classification groups are:

    - Ground (Project Area)

    - Roadway

    - Utilities (Signs, Utility Poles, Utility Lines)

    - Feature of Interest (Bridge, Culvert, Ledge, etc).

- 0.5ft Sampled Surface DGN

  - DXF (include cloud and geometries in export) is merged into DGN

- 1.0ft Sampled Surface DGN

  - DXF (include cloud and geometries in export) is merged into DGN

- Digitized Features DGN

  - DXF (geometries only) is merged into DGN

Only POD files can be attached to a MicroStation DGN since it is the only format that can be natively used by the software’s point cloud engine - Pointools. For instructions on how to view point clouds in MicroStation see Working with POD Files in MicroStation. The exported deliverables formats should be noted in the STLS Processing Form.

If a surface is requested as a deliverable for in-house use at VTrans, the exported DXF surfaces will need to be merged and saved as a DGN. The DXF unit must be exported as US Survey Feet from Trimble RealWorks, attached as a reference in a blank MicroStation DGN (with vtseed3d.dgn as seed), merged with the DGN, and saved as a .DGN file. The DGN(s) should be what is delivered as the surface file. For details on this process please see the DXF to DGN Surface Deliverable Workflow documentation.

If the project is associated to a programmed project, the exported deliverables should be uploaded into a .ZIP in a LiDAR folder on the M:Drive (M:/Projects/PIN/Survey/LiDAR). If the project is unprogrammed, exported deliverables will be kept in .ZIP in a Exports folder within the project folder on the L:Drive or appropriately uploaded to different server locations or FTP sites.

### Documentation

##### Survey Control Files

Survey control values used in georeferencing must be uploaded as a .TXT to the Survey folder for the specific project. If a conventional survey was also performed for the project, please include a shortcut to the M:Drive folder location or a copy of the DGN in the Survey folder on the L:Drive.

##### STLS Processing Form

This Survey123 form is used to document each project request and processing procedure. When a request comes in, a form entry must be started and include the appropriate scan and request information for the project. While processing a project, this form must be updated to include appropriate processing information. Upon project completion, the processer must ensure that the all the appropriate fields has been filled out and update the project completion date.

STLS Processing Entry Form:

https://survey123.arcgis.com/share/de6feb940200436aafe168b6abddec6d

To edit existing form entries (to update processing information or completion date) use the Form Editor page. You will need access to your ArcOnline account log in to access this page. This site allows you to interact with all the scan project entries and update project attribute information. It is recommended that all editing be performed in the form format.

After selecting the entry to be edited by either clicking on the map feature or in the table, click the editing pencil highlighted in the image above and it will open the Form Format. This allows you to use all the customized form questions and upload photo attachments.

Survey123 Form Editor Site:

https://arcg.is/SO1aL

By completing the form, the processer certifies all the following files have been included and the deliverables adhere to data requirements and standards:

- Scan Field Notes

- RTF Registration Error Report

- Survey Control Files (or file path specifying location on different State server)